M3H Communication Protocol-B600

1 Communication format

1.1 baud rate

9600

1.2 data format

8, n. 1

1.3 data frame format

ASCII

2 Communication terminal

1) main control equipment computer

2) controlled device controller

3 Instruction type

3.1 computer sends controller response instructions

3.1.1 control instructions

3.1.2 setting instructions

3.1.3 query instruction

3.1.4 reset instruction

3.2 the controller sends instructions actively

The controller sends the motor status instruction actively

4 Computer sends controller response instruction

The computer sends instructions to the controller actively. After receiving the instructions, the controller determines whether the instructions are correct. If correct,the controller performs corresponding operations and sends response data to the computer.If exceptional,the controller directly ignored.

4.1 the computer controls the motor to run step by step

4.1.1 fixed-length instruction (fixed-length instruction is mainly compatible with the old system)

Computer send: @x000 +12345678ww!

Controller response: @x000 +12345678!

| number | definition | content | remarks |
| --- | --- | --- | --- |
| 1 | Start of frame | @ |  |
| 2 | Specified motor | X | control X-axis motor |
|  |  | Y | control Y-axis motor |
|  |  | U | simultaneously controls X and Y axis motors |
|  |  | V | simultaneously controls X and Y axis motors |
|  |  | Z | controls the z-axis motor |
| 3-5 | Motor speed | 000 - 999 |  |
| 6 | Motor forward boundary | + | forward operation |
|  |  | - | reverse operation |
| 7-14 | Number of running steps | 00000000-99999999 |  |
| 15-16 | Undefined | Can not be `@’ `!’ | The controller responds: there is no this part |
| 17 | Frame tail | ！ |  |

Note 1: U+ corresponds to the forward operation of the XY axis motor, while U- corresponds to the reverse operation of the XY axis motor;

Note 2: V+ corresponds to the forward operation of the X-axis motor and the reverse operation of the Y-axis motor, while V- corresponds to the reverse operation of the X-axis motor and the reverse operation of the Y-axis motor.

4.1.2 Reduced instruction

Computer send:   @X000+12345678!

Controller response：@X000+12345678!

| number | definition | content | remarks |
| --- | --- | --- | --- |
| 1 | Start of frame | @ |  |
| 2 | Specified motor | X | control X-axis motor |
|  |  | Y | control Y-axis motor |
|  |  | U | simultaneously controls X and Y axis motors |
|  |  | V | simultaneously controls X and Y axis motors |
|  |  | Z | controls the z-axis motor |
| 3-5 | Motor speed | 000 - 999 |  |
| 6 | Motor forward boundary | + | forward operation |
|  |  | - | reverse operation |
| 7-14 | Number of running steps | 00000000-99999999 |  |
| 15 | Frame tail | ！ |  |

4.2 Computer controls the motor to run to the boundary command

4.2.1 fixed-length instruction

Computer send：

@X000A12345678ww!

Controller response：

@X000A!

| number | definition | content | remarks |
| --- | --- | --- | --- |
| 1 | Start of frame | @ |  |
| 2 | Specified motor | X | control X-axis motor |
|  |  | Y | control Y-axis motor |
|  |  | U | simultaneously controls X and Y axis motors |
|  |  | V | simultaneously controls X and Y axis motors |
|  |  | Z | control X-axis motor |
| 3-5 | Motor speed | 000 - 999 |  |
| 6 | Motor forward boundary | A | forward operation |
|  |  | B | reverse operation |
| 7-16 | Undefined | Can not be`@’ `!’ | The controller responds: there is no this part |
| 17 | Frame tail | ！ |  |

4.2.2 Reduced instruction

Computer send：

@X000A!

Controller response：

@X000A!

| number | definition | content | remarks |
| --- | --- | --- | --- |
| 1 | Start of frame | @ |  |
| 2 | Specified motor | X | control X-axis motor |
|  |  | Y | control Y-axis motor |
|  |  | U | simultaneously controls X and Y axis motors |
|  |  | V | simultaneously controls X and Y axis motors |
|  |  | Z | controls the z-axis motor |
| 3-5 | Motor speed | 000 - 999 |  |
| 6 | Motor forward boundary | A | forward operation |
|  |  | B | reverse operation |
| 7 | Frame tail | ！ |  |

4.3 Computer control specified motor stop instruction

4.3.1 fixed-length instruction

Computer send：

@X000F12345678ww!

Controller response：

@XF!

| number | definition | content | remarks |
| --- | --- | --- | --- |
| 1 | Start of frame | @ |  |
| 2 | Specified motor | X | control X-axis motor |
|  |  | Y | control Y-axis motor |
|  |  | U | simultaneously controls X and Y axis motors |
|  |  | V | simultaneously controls X and Y axis motors |
|  |  | Z | controls the z-axis motor |
| 3-5 | Motor speed | 000 - 999 | The controller responds: there is no this part |
| 6 | Motor forward boundary | F | The motor can stop freely and rotate manually |
|  |  | S | The motor stops and locks,can not be rotated manually |
| 7-16 | Undefined | Can not be`@’ `!’ | The controller responds: there is no this part |
| 17 | Frame tail | ！ |  |

4.3.2 Reduced instruction

Computer send：

@XF!

Controller response：

@XF!

| number | definition | content | remarks |
| --- | --- | --- | --- |
| 1 | Start of frame | @ |  |
| 2 | Specified motor | X | control X-axis motor |
|  |  | Y | control Y-axis motor |
|  |  | U | simultaneously controls X and Y axis motors |
|  |  | V | simultaneously controls X and Y axis motors |
|  |  | Z | controls the z-axis motor |
| 3 | Motor forward boundary | F | The motor can stop freely and rotate manually |
|  |  | S | The motor stops and locks,can not be rotated manually |
| 4 | Frame tail | ！ |  |

4.4 Computers control global commands

4.4.1 fixed-length instruction

Computer send：

@CF00012345678ww!

Controller response：

@CF!

| number | definition | content | remarks |
| --- | --- | --- | --- |
| 1 | Start of frame | @ |  |
| 2 | Identifier | C |  |
| 3 | Instruction | F | All motor can stop freely and rotate manually |
|  |  | S | All motor stops and locks,can not be rotated manually |
|  |  | P | The controller goes into jigsaw mode |
|  |  | Q | The controller exits jigsaw mode |
| 4-16 | Undetified | Can not be`@’ `!’ | The controller responds: there is no this part |
| 17 | Frame tail | ！ |  |

4.4.2  Reduced instruction

Computer send：

@CF!

Controller response：

@CF!

| number | definition | content | remarks |
| --- | --- | --- | --- |
| 1 | Start of frame | @ |  |
| 2 | Identifier | C |  |
| 3 | Instruction | F | All motor can stop freely and rotate manually |
|  |  | S | All motor stops and locks,can not be rotated manually |
|  |  | P | The controller goes into jigsaw mode |
|  |  | Q | The controller exits jigsaw mode |
| 4 | Frame tail | ！ |  |

# 5 Parameter setting instruction

5.1 Computer sends instructions to set X, Y and Z motor parameters

Computer send：

@x+-FX001020Y300!

Controller response：

@x+-FX001020Y300!

After receiving the setting instruction, the controller determines that the setting instruction effectively responds to the same data frame, and if it is invalid, it does not respond.

| number | definition | content | remarks |
| --- | --- | --- | --- |
| 1 | Start of frame | @ |  |
| 2 | Specified motor | x | corresponding X-axis motor,lowercase letters |
|  |  | y | corresponding Y-axis motor,lowercase letters |
|  |  | z | corresponding Z-axis motor,lowercase letters |
| 3 | Motor forward boundary | + | Forward boundary works |
|  |  | N | Forward boundary does not work |
| 4 | Motor reverse boundary | - | Reverse boundary works |
|  |  | N | Reverese boundary does not work |
| 5 | Motor stops after runing | F | The motor is free and can rotate manually |
|  |  | S | The motor locks,can not be rotated manually |
| 6 | Rocker mapping | X | The corresponding rocker is set to control the X motor |
|  |  | Y | The corresponding rocker is set to control the Y motor |
|  |  | Z | The corresponding rocker is set to control the Z motor |
|  |  | N | The corresponding rocker does not work |
| 7 | Rocker control high speed | 000 - 999 |  |
| 8 |  |  |  |
| 9 |  |  |  |
| 10 | Rocker control low speed | 000 - 999 |  |
| 11 |  |  |  |
| 12 |  |  |  |
| 13 | Motor start delay | Y | Motor start delay enable |
|  |  | N | Motor start delay shutdown |
| 14 |  | 000-999 | Motor start delay parameters |
| 15 |  |  |  |
| 16 |  |  |  |
| 17 | Frame tail | ！ |  |

Note: if the response data frame is inconsistent with the set data frame, the controller may have storage failure!

5.2 The computer sends instructions to set up the speed mapping

Computer send：

@TX000=F123!

Controller response：

@TX000=F123!

After receiving the setting instruction, the controller determines that the setting instruction effectively responds to the same data frame, and if it is invalid, it does not respond.

| number | definition | content | remarks |
| --- | --- | --- | --- |
| 1 | Start of frame | @ |  |
| 2 | Identifier | T |  |
| 3 | Motor | X |  |
|  |  | Y |  |
|  |  | Z |  |
| 4 | Speed | 000-999 |  |
| 5 |  |  |  |
| 6 |  |  |  |
| 7 | Identifier | = |  |
| 8 | Setting value | 0000 - FFFF |  |
| 9 |  |  |  |
| 10 |  |  |  |
| 11 |  |  |  |
| 12 | Frame tail | ！ |  |

# 6 Parameter reset instruction

Computer send：

@Rx!

Controller response：

@Rx!

After receiving the setting instruction, the controller determines that the setting instruction effectively responds to the same data frame, and if it is invalid, it does not respond.

| number | definition | content | remarks |
| --- | --- | --- | --- |
| 1 | Start of frame | @ |  |
| 2 | Identifier | R |  |
| 3 | Instruction | x | Reset X motor parameters |
|  |  | y | Reset Y motor parameters |
|  |  | z | Reset Z motor parameters |
|  |  | T | Reset motor speed mapping parameters |
|  |  | X | Reset relative position of X motor |
|  |  | Y | Reset relative position of Y motor |
|  |  | P | Reset relative position of X and Y motor |
|  |  | Z | Reset relative position of Z motor |
|  |  | M | Reset all system parameters |
| 4 | Frame tail | ！ |  |

# 7 Query motor parameter instruction

7.1 Query X,Y,Z motor parameter instruction （Query  5.1 setting parameters）

Computer send：

@Ax!

Controller response：

@Ax: x+-FX001020Y300!

After receiving the setting instruction, the controller determines that the setting instruction effectively responds to the same data frame, and if it is invalid, it does not respond.

7.1.1 The computer sends the instruction of querying motor parameter

| number | definition | content | remarks |
| --- | --- | --- | --- |
| 1 | Start of frame | @ |  |
| 2 | Identifier | A |  |
| 3 | Insturction | x | X motor parameters |
|  |  | y | Y motor parameters |
|  |  | z | Z motor parameters |
| 4 | Frame tail | ！ |  |

7.1.2 Controller response

| number | definition | content | remarks |
| --- | --- | --- | --- |
| 1 | Start of frame | @ |  |
| 2-4 | Identifier | Ax: | corresponding X-axis motor |
|  |  | Ay: | correspondingY-axis motor |
|  |  | Az: | Corresponding Z-axis motor |
| 5 | Specified motor | X | corresponding X-axis motor |
|  |  | Y | correspondingY-axis motor |
|  |  | Z | Corresponding Z-axis motor |
| 6 | Motor forward boundary | + | Forward boundary works |
|  |  | N | Forward boundary does not work |
| 7 | Motor reverse boundary | - | Reverse boundary works |
|  |  | N | Reverese boundary does not work |
| 8 | Motor stops after runing | F | The motor is free and can rotate manually |
|  |  | S | The motor locks,can not be rotated manually |
| 9 | Rocker mapping | X | The corresponding rocker is set to control the X motor |
|  |  | Y | The corresponding rocker is set to control the Y motor |
|  |  | Z | The corresponding rocker is set to control the Z motor |
|  |  | N | The corresponding rocker does not work |
| 10 | Rocker control high speed | 000 - 999 |  |
| 11 |  |  |  |
| 12 |  |  |  |
| 13 | Rocker control low speed | 000 - 999 |  |
| 14 |  |  |  |
| 15 |  |  |  |
| 16 | Motor start delay | Y | Motor start delay enable |
|  |  | N | Motor start delay shutdown |
| 17 |  | 000-999 | Motor start delay parameters |
| 18 |  |  |  |
| 19 |  |  |  |
| 20 | Frame tail | ！ |  |

# 7.2 Query speed mapping parameter instruction（Query  5.2 setting parameters）

Computer send：

@tX000!

Controller response：

@tX000=F123!

After receiving the setting instruction, the controller determines that the setting instruction effectively responds to the same data frame, and if it is invalid, it does not respond.

7.2.1 Computer sends instructions for speed mapping parameters

| number | definition | content | remarks |
| --- | --- | --- | --- |
| 1 | Start of frame | @ |  |
| 2 | Identifier | t |  |
| 3 | Motor | X |  |
|  |  | Y |  |
|  |  | Z |  |
| 4 | Speed | 000-999 |  |
| 5 |  |  |  |
| 6 |  |  |  |
| 7 | Frame tail | ！ |  |

7.2.2 Controller response

| number | definition | content | remarks |
| --- | --- | --- | --- |
| 1 | Start of frame | @ |  |
| 2 | Identifier | t |  |
| 3 | Motor | X |  |
|  |  | Y |  |
|  |  | Z |  |
| 4 | Speed | 000-999 |  |
| 5 |  |  |  |
| 6 |  |  |  |
| 7 | Identifier | = |  |
| 8 | Setting value | 0000 - FFFF |  |
| 9 |  |  |  |
| 10 |  |  |  |
| 11 |  |  |  |
| 12 | Frame tail | ！ |  |

# 7.3  Query program version

Computer send：

@Aa!

Controller response：

@ Aa:M3H-S30M-B600!

After receiving the setting instruction, the controller determines that the setting instruction effectively responds to the corresponding data frame, and if it is invalid, it does not respond.

7.3.1 Computer sends instructions for speed mapping parameters

| number | definition | content | remarks |
| --- | --- | --- | --- |
| 1 | Start of frame | @ |  |
| 2-3 | Identifier | Aa |  |
| 4 | Frame tail | ！ |  |

7.3.2  Controller response

| number | definition | content | remarks |
| --- | --- | --- | --- |
| 1 | Start of frame | @ |  |
| 2-4 | Identifier | Aa: |  |
| 5-12 | Program name | M3H-S30M |  |
| 13 | Identifier | - |  |
| 14-18 | Version number | B6000 |  |
| 19 | Frame tail | ！ |  |

# 8 Status query instruction

8.1 Query motor status instruction

Computer send：

@!

Controller response：

@0!

8.1.1 Computer send

|  | Start of frame | Frame tail |
| --- | --- | --- |
| Byte | 1 | 2 |
| Quick query point motor status | @ | ！ |

8.1.2 Controller response

| Data frames | Z | Y | X |
| --- | --- | --- | --- |
| @0！ | stop | stop | stop |
| @1！ | stop | stop | operate |
| @2！ | stop | operate | stop |
| @3！ | stop | operate | operate |
| @4！ | operate | stop | stop |
| @5！ | operate | stop | operate |
| @6！ | operate | operate | stop |
| @7！ | operate | operate | operate |
| Else | abnormal |  |  |

8.2  Query motor position instruction

8.2.1 Query the position of individual motor

Computer send：@x!

Controller response：@Mx:X000A0=12345678,+12345678!

After receiving the setting instruction, the controller determines that the setting instruction effectively responds to the corresponding data frame, and if it is invalid, it does not respond.

8.2.1.1    Computer send

| number | definition | content | remarks |
| --- | --- | --- | --- |
| 1 | Start of frame | @ |  |
| 2 | Motor | x | X axis motor |
|  |  | y | Y axis motor |
|  |  | z | Z axis motor |
| 3 | Frame tail | ！ |  |

8.2.1.2 Controller response the position of individual  X Y Z  motor

| number | definition | content | remarks |
| --- | --- | --- | --- |
| 1 | Start of frame | @ |  |
| 2-4 | Identifier | Mx: | corresponding X-axis motor |
|  |  | My: | correspondingY-axis motor |
|  |  | Mz: | corresponding Z-axis motor |
| 5 | Specified motor | X | corresponding X-axis motor |
|  |  | Y | correspondingY-axis motor |
|  |  | Z | corresponding Z-axis motor |
| 6-8 | Speed | 000 - 999 |  |
| 9 | Status | F | stop free |
|  |  | S | stop and lock |
|  |  | A | run to forward boundry |
|  |  | B | run to reverse boundry |
|  |  | + | forward run in steps |
|  |  | - | reverse run in steps |
| 10 | Boundry | 0 | within the boundry |
|  |  | 1 | forward boundry |
|  |  | 2 | reverse boundry |
|  |  | 3 | abnormal boundry |
| 11 | Identifier | = |  |
| 12-19 |  | 0000000 - 99999999 | motor remaining steps |
| 20 | Identifier | , |  |
| 21-29 |  | -99999999 - +99999999 | relative position |
| 30 | Frame tail | ！ |  |

8.2.2 Query all motors position

Computer send：

@m!

Cotroller response：

@Mm:X000A0=12345678,+12345678;Y000A0=12345678,+12345678;Z000A0=12345678,+12345678!

After receiving the setting instruction, the controller determines that the setting instruction effectively responds to the corresponding data frame, and if it is invalid, it does not respond.

8.2.2.1  Computer send

| number | definition | content | remarks |
| --- | --- | --- | --- |
| 1 | Start of frame | @ |  |
| 2 | Motor | m | X Y Z axis motor |
| 3 | Frame tail | ！ |  |

8.2.2.2 Cotroller response all the motors position

| number | definition | content | remarks |
| --- | --- | --- | --- |
| 1 | Start of frame | @ |  |
| 2-4 | Identifier | Mm: | corresponding X Y Z axis motor |
| 5 | Specified motor | X | corresponding X-axis motor |
| 6-8 | Speed | 000 - 999 |  |
| 9 | Status | F | stop free |
|  |  | S | stop and lock |
|  |  | A | run to forward boundry |
|  |  | B | run to reverse boundry |
|  |  | + | forward run in steps |
|  |  | - | reverse run in steps |
| 10 | Boundry | 0 | within the boundry |
|  |  | 1 | forward boundry |
|  |  | 2 | reverse boundry |
|  |  | 3 | abnormal boundry |
| 11 | Identifier | = |  |
| 12-19 |  | 0000000 - 99999999 | motor remaining steps |
| 20 | Identifier | , |  |
| 21-29 |  | -99999999 - +99999999 | motor relative position |
| 30 | Identifier | ; |  |
| 31 | Specified motor | Y | corresponding Z-axis motor |
| 32-34 | Speed | 000 - 999 |  |
| 35 | Status | F | stop free |
|  |  | S | stop and lock |
|  |  | A | run to forward boundry |

|  |  | B | run to reverse boundry |
| --- | --- | --- | --- |
|  |  | + | forward run in steps |
|  |  | - | reverse run in steps |
| 36 | Boundry | 0 | within the boundry |
|  |  | 1 | forward boundry |
|  |  | 2 | reverse boundry |
|  |  | 3 | abnormal boundry |
| 37 | Identifier | = |  |
| 38-45 |  | 0000000 - 99999999 | motor remaining steps |
| 46 | Identifier | , |  |
| 47-55 |  | -99999999 - +99999999 | motor relative position |
| 56 | Identifier | ; |  |
| 57 | Specified motor | Z | corresponding Z-axis motor |
| 58-60 | Speed | 000 - 999 |  |
| 61 | Status | F | stop free |
|  |  | S | stop and lock |
|  |  | A | run to forward boundry |
|  |  | B | run to reverse boundry |
|  |  | + | forward run in steps |
|  |  | - | reverse run in steps |
| 62 | Boundry | 0 | within the boundry |
|  |  | 1 | forward boundry |
|  |  | 2 | reverse boundry |
|  |  | 3 | abnormal boundry |
| 63 | Identifier | = |  |
| 64-71 |  | 0000000 - 99999999 | motor remaining steps |
| 72 | Identifier | , |  |
| 73-81 |  | -99999999 - +99999999 | motor relative position |
| 82 | Frame tail | ! |  |

8.2.3  Query jigsaw position

Computer send：

@p!

Controller response：

@Mp: +12345678+12345678!

After receiving the setting instruction, the controller determines that the setting instruction effectively responds to the corresponding data frame, and if it is invalid, it does not respond.

8.2.3.1  Computer send

| number | definition | content | remarks |
| --- | --- | --- | --- |
| 1 | Start of frame | @ |  |
| 2 | Motor | m | X Y Z axis motor |
| 3 | Frame tail | ！ |  |

8.2.3.2 Controller response

| number | definition | content | remarks |
| --- | --- | --- | --- |
| 1 | Start of frame | @ |  |
| 2-4 | Identifier | Mp: | corresponding X-axis motor |
| 5-13 | X-axis motor position | -99999999 - +99999999 |  |
| 14 | Identifier | , |  |
| 15-23 | X-axis motor position | -99999999 - +99999999 |  |
| 24 | Frame tail | ！ |  |

# 9  The controller actively sends the motor stop instruction

Note: this instruction only runs in jigsaw mode

| Data frames | Z | Y | X |
| --- | --- | --- | --- |
| @0！ | stop | stop | stop |
| @1！ | stop | stop | operate |
| @2！ | stop | operate | stop |
| @3！ | stop | operate | operate |
| @4！ | operate | stop | stop |
| @5！ | operate | stop | operate |
| @6！ | operate | operate | stop |
| @7！ | operate | operate | operate |
| Else | abnormal |  |  |